This is a very common question in Dynatrace, especially with PostgreSQL. The short answer is:

The Database Service response time includes much more than just the SQL execution time.

The SQL Statement response time measures only the execution of an individual SQL statement, while the Database Service overview measures the end-to-end time spent interacting with the database service.

For example, suppose your application executes one query:

Application

|

|---- Acquire connection (20 ms)

|---- Wait in connection pool (50 ms)

|---- Send SQL to PostgreSQL (5 ms)

|---- PostgreSQL executes SQL (40 ms)

|---- Transfer result set (30 ms)

|---- Deserialize/process rows (15 ms)

|

Total Database Service = 160 ms

But the SQL statement itself only takes:

SQL execution = 40 ms

So Dynatrace might display:

- Database Service response time: 160 ms

- SQL Statement response time: 40 ms

## Common reasons for the difference

### 1. Connection pool waiting (very common)

If you use HikariCP, which you've mentioned before, the application may wait for a free database connection.

Example:

- Wait for connection: 120 ms

- SQL execution: 15 ms

Database Service = 135 ms

SQL Statement = 15 ms

### 2. Network latency

The Database Service includes the round-trip time between the application and PostgreSQL.

Application

|

| 10 ms

|

PostgreSQL

|

| 10 ms

A 15 ms SQL becomes about 35 ms from the application's perspective.

### 3. Large result sets

For example:

SELECT *

FROM documents;

Execution:

- PostgreSQL executes query in 12 ms

But:

- returns 100 MB

- application reads 200,000 rows

Total Database Service = 400 ms

SQL Statement = 12 ms

### 4. JDBC driver processing

The JDBC driver spends time:

- reading packets

- decoding data

- converting PostgreSQL types to Java types

This time is not SQL execution.

### 5. Transaction overhead

For example:

BEGIN

SELECT ...

UPDATE ...

COMMIT

The Database Service may include transaction-related work, while each SQL statement is measured separately.

### 6. Connection establishment

If connections are not reused:

TCP handshake

SSL handshake

Authentication

This time contributes to the Database Service but not to SQL execution.

## PostgreSQL-specific reasons

Since you're using PostgreSQL, also consider:

- SSL encryption

- slow client reads

- large BYTEA columns (I remember you mentioned tables containing BYTEA)

- server sending many rows

- application consuming rows slowly

A query can execute in:

20 ms

but take

250 ms

to transmit the results.

## How to verify in Dynatrace

Open:

Database Service → Analyze → Requests

Compare:

Database Service Response Time

with

SQL Statement Execution Time

If the difference is large, inspect:

- Connection acquisition time

- Network time

- Result size

- Number of returned rows

- Connection pool (HikariCP) metrics

- PostgreSQL wait events (pg_stat_activity)

- PostgreSQL I/O statistics (pg_stat_io on PostgreSQL 16+)

## Based on your previous questions

I remember you mentioned:

- PostgreSQL

- HikariCP

- BYTEA columns

- Dynatrace response time discrepancies

In that environment, the most common causes are:

- Waiting for a HikariCP connection.

- Large BYTEA values being transferred.

- Slow client-side reading of the result set.

- Network latency between the application pod and PostgreSQL.

If your Database Service response time is 2–10× higher than the SQL statement time, these are the first areas I would investigate.

If you can share:

- the Database Service response time,

- the SQL statement response time,

- the PostgreSQL version,

- and whether the application runs on Kubernetes/OpenShift,

I can help determine which component is responsible for the extra latency.